# Drawing Standards Manual

## 4th Dimension Architects

## 1. Purpose

This manual establishes the drawing standards for all architectural documentation at 4th Dimension Architects. Adherence ensures consistency, professionalism, and efficient collaboration.

## 2. Drawing Sizes & Scales

### Standard Sheet Sizes

| Size | Dimensions (mm) | Use |
| --- | --- | --- |
| A0 | 841 x 1189 | Master plans, large projects |
| A1 | 594 x 841 | Primary working drawings |
| A2 | 420 x 594 | Detail sheets |
| A3 | 297 x 420 | Client presentations, reports |

### Standard Scales

| Drawing Type | Preferred Scale | Alternative |
| --- | --- | --- |
| Site Plan | 1:500 | 1:200 |
| Floor Plans | 1:100 | 1:50 |
| Elevations | 1:100 | 1:50 |
| Sections | 1:50 | 1:100 |
| Details | 1:20, 1:10 | 1:5 |
| Furniture Layout | 1:50 | 1:100 |

## 3. Title Block

### Required Information

- Company logo and name

- Project name and code

- Drawing title

- Drawing number

- Scale

- Date

- Drawn by / Checked by / Approved by

- Revision table

- Sheet number (X of Y)

### Title Block Location

- Always bottom-right corner

- Standard size: 180mm x 60mm

## 4. Layer Naming Convention

### Format: DISCIPLINE-ELEMENT-STATUS

### Disciplines

| Code | Discipline |
| --- | --- |
| A | Architectural |
| S | Structural |
| M | Mechanical |
| E | Electrical |
| P | Plumbing |
| L | Landscape |
| I | Interior |

### Elements

| Code | Element |
| --- | --- |
| WALL | Walls |
| DOOR | Doors |
| WIND | Windows |
| FURN | Furniture |
| CLNG | Ceiling |
| FLOR | Flooring |
| DIMS | Dimensions |
| TEXT | Text/Notes |
| HATCH | Hatching |
| GRID | Grid lines |

### Status

| Code | Meaning |
| --- | --- |
| NEW | New work |
| EXIST | Existing |
| DEMO | To be demolished |
| FUT | Future work |

### Examples

- A-WALL-NEW - New architectural walls

- A-DOOR-EXIST - Existing doors

- I-FURN-NEW - New furniture

- A-WALL-DEMO - Walls to be demolished

## 5. Line Types & Weights

### Line Weights

| Weight | mm | Use |
| --- | --- | --- |
| Heavy | 0.50 | Section cuts, outlines |
| Medium | 0.35 | Major elements, walls |
| Light | 0.25 | Furniture, text |
| Fine | 0.18 | Hatching, dimensions |
| Hairline | 0.13 | Construction lines |

### Line Types

| Type | Use |
| --- | --- |
| Continuous | Visible outlines |
| Dashed | Hidden lines, above cut |
| Chain | Center lines, grid |
| Dotted | Demolished/removed |

## 6. Text Standards

### Font

- Primary: Arial or Simplex

- Titles: Arial Bold

### Text Heights

| Use | Height (mm) |
| --- | --- |
| Drawing title | 5.0 |
| Room names | 3.5 |
| Dimensions | 2.5 |
| Notes | 2.5 |
| Fine annotations | 2.0 |

## 7. Dimension Standards

### Rules

- Always dimension to finished surfaces

- Use running dimensions where possible

- Maintain consistent dimension line spacing (10mm)

- Place dimensions outside the drawing when possible

### Dimension Styles

- Arrow type: Architectural tick or oblique

- Text position: Above line, centered

- Units: Millimeters (no suffix)

## 8. Hatching Standards

### Common Patterns

| Material | Pattern | Scale |
| --- | --- | --- |
| Brick | ANSI31 | 1.0 |
| Concrete | AR-CONC | 0.5 |
| Earth | EARTH | 1.0 |
| Insulation | INSUL | 1.0 |
| Wood | WOOD | 1.0 |
| Glass | Solid | - |

## 9. Drawing Numbering

### Format: PROJECT-DISCIPLINE-TYPE-NUMBER

### Example: 2024-001-A-FP-01

- 2024-001: Project code

- A: Architectural

- FP: Floor Plan

- 01: Sheet number

### Drawing Types

| Code | Type |
| --- | --- |
| LOC | Location plan |
| SP | Site plan |
| FP | Floor plan |
| RCP | Reflected ceiling plan |
| EL | Elevation |
| SEC | Section |
| DET | Details |
| SCH | Schedules |
| 3D | 3D views |

## 10. Revision Management

### Revision Clouds

- Use revision clouds to highlight changes

- Triangle marker with revision letter

### Revision Table

| Rev | Date | Description | By |
| --- | --- | --- | --- |
| A | 01/01/25 | Issued for Review | DS |
| B | 15/01/25 | Client comments | AK |

### Revision Naming

- Pre-issue: P1, P2, P3…

- Issued: A, B, C…

## 11. File Naming Convention

### Format

[ProjectCode]-[Discipline]-[Type]-[Number]-[Rev]

### Example

2024-001-A-FP-01-B.dwg

## 12. Quality Checklist Before Issue

- ☐ All layers correctly named

- ☐ Title block complete and accurate

- ☐ Dimensions verified

- ☐ Text spell-checked

- ☐ Hatching consistent

- ☐ File purged (PURGE command)

- ☐ Xrefs bound (for issue)

- ☐ PDF generated correctly

- ☐ Reviewed by Team Leader

Standard effective from: January 2025 Review annually