# CAD Layer Standards

## AutoCAD Layer Naming Convention

### 4th Dimension Architects

## Layer Naming Format

DISCIPLINE-ELEMENT-STATUS

## Complete Layer List

### Architectural Layers (A-)

| Layer Name | Color | Lineweight | Description |
| --- | --- | --- | --- |
| A-WALL-NEW | 7 (White) | 0.35 | New walls |
| A-WALL-EXIST | 8 (Gray) | 0.25 | Existing walls |
| A-WALL-DEMO | 1 (Red) | 0.25 | Walls to demolish |
| A-WALL-PART | 7 | 0.25 | Partition walls |
| A-DOOR-NEW | 3 (Green) | 0.25 | New doors |
| A-DOOR-EXIST | 8 | 0.18 | Existing doors |
| A-WIND-NEW | 5 (Blue) | 0.25 | New windows |
| A-WIND-EXIST | 8 | 0.18 | Existing windows |
| A-STAIR | 7 | 0.35 | Staircases |
| A-RAMP | 7 | 0.25 | Ramps |
| A-LIFT | 7 | 0.25 | Elevators |
| A-ROOF | 7 | 0.25 | Roof elements |
| A-CLNG | 4 (Cyan) | 0.18 | Ceiling |
| A-FLOR | 7 | 0.18 | Floor finishes |
| A-SLAB | 7 | 0.35 | Floor slab |
| A-BEAM | 7 | 0.25 | Beams (exposed) |
| A-COLS | 7 | 0.35 | Columns |
| A-BALC | 7 | 0.25 | Balcony |
| A-RAIL | 7 | 0.18 | Railings |

### Interior Layers (I-)

| Layer Name | Color | Lineweight | Description |
| --- | --- | --- | --- |
| I-FURN | 6 (Magenta) | 0.18 | Furniture |
| I-FURN-SOFT | 6 | 0.13 | Soft furnishing |
| I-EQPM | 6 | 0.18 | Equipment |
| I-APPL | 6 | 0.18 | Appliances |
| I-FIXT | 6 | 0.18 | Fixtures |
| I-ACCS | 6 | 0.13 | Accessories |
| I-CASE | 6 | 0.18 | Casework/Built-in |
| I-PLNT | 3 | 0.13 | Plants |
| I-ART | 6 | 0.13 | Artwork |

### Annotation Layers

| Layer Name | Color | Lineweight | Description |
| --- | --- | --- | --- |
| A-DIMS | 2 (Yellow) | 0.18 | Dimensions |
| A-TEXT | 7 | 0.18 | General text |
| A-NOTE | 7 | 0.18 | Notes |
| A-SYMB | 7 | 0.18 | Symbols |
| A-LEVL | 2 | 0.18 | Level markers |
| A-GRID | 8 | 0.13 | Grid lines |
| A-MATCH | 3 | 0.18 | Match lines |
| A-SECT | 1 | 0.25 | Section markers |
| A-DETL | 1 | 0.25 | Detail markers |
| A-ROOM | 4 | 0.18 | Room names/numbers |
| A-AREA | 4 | 0.18 | Area calculations |
| A-TAGS | 7 | 0.18 | Door/window tags |
| A-NORTH | 7 | 0.25 | North arrow |
| A-SCALE | 7 | 0.18 | Scale bars |

### Hatching Layers

| Layer Name | Color | Lineweight | Description |
| --- | --- | --- | --- |
| A-HATCH-SOLID | 8 | 0.13 | Solid fill |
| A-HATCH-WALL | 8 | 0.13 | Wall hatch |
| A-HATCH-CONC | 8 | 0.13 | Concrete hatch |
| A-HATCH-BRICK | 8 | 0.13 | Brick hatch |
| A-HATCH-WOOD | 8 | 0.13 | Wood hatch |
| A-HATCH-TILE | 8 | 0.13 | Tile hatch |
| A-HATCH-EARTH | 8 | 0.13 | Earth/soil |
| A-HATCH-INSUL | 8 | 0.13 | Insulation |

### Site Layers (L-)

| Layer Name | Color | Lineweight | Description |
| --- | --- | --- | --- |
| L-PROP | 7 | 0.35 | Property line |
| L-SETB | 2 | 0.25 | Setback lines |
| L-ROAD | 8 | 0.25 | Roads |
| L-PAVE | 8 | 0.18 | Paving |
| L-PLNT | 3 | 0.18 | Planting |
| L-TREE | 3 | 0.18 | Trees |
| L-LAWN | 3 | 0.13 | Lawn area |
| L-CONT | 8 | 0.13 | Contours |
| L-BLDG-ADJ | 8 | 0.18 | Adjacent buildings |

### Electrical Layers (E-)

| Layer Name | Color | Lineweight | Description |
| --- | --- | --- | --- |
| E-LITE | 2 | 0.18 | Lighting |
| E-POWR | 1 | 0.18 | Power outlets |
| E-SWCH | 2 | 0.18 | Switches |
| E-DATA | 5 | 0.18 | Data/telecom |
| E-SECU | 1 | 0.18 | Security |

### Plumbing Layers (P-)

| Layer Name | Color | Lineweight | Description |
| --- | --- | --- | --- |
| P-FIXT | 5 | 0.18 | Plumbing fixtures |
| P-PIPE | 5 | 0.18 | Pipes |
| P-EQPM | 5 | 0.18 | Equipment |

### Utility Layers

| Layer Name | Color | Lineweight | Description |
| --- | --- | --- | --- |
| X-BORDER | 7 | 0.50 | Drawing border |
| X-TITLE | 7 | 0.35 | Title block |
| X-VPRT | 9 | 0.13 | Viewport frame |
| X-XREF | 8 | 0.13 | Xref only |
| X-NPLT | 9 | 0.00 | Non-plot |
| DEFPOINTS | - | - | AutoCAD default |

## Color Chart Reference

| Number | Color | Typical Use |
| --- | --- | --- |
| 1 | Red | Demo, sections |
| 2 | Yellow | Dimensions, electrical |
| 3 | Green | Doors, planting |
| 4 | Cyan | Ceiling, rooms |
| 5 | Blue | Windows, plumbing |
| 6 | Magenta | Interior/furniture |
| 7 | White/Black | Primary elements |
| 8 | Gray | Existing, background |
| 9 | Light Gray | Non-plot, viewport |

## Best Practices

- Never use Layer 0 for drawing objects

- Always freeze unused layers rather than turning off

- Lock reference layers to prevent accidental edits

- Purge unused layers before issue

- Use layer filters for efficient layer management

Reference: Based on AIA CAD Layer Guidelines and NCS v6.0